Speaker Verification

Using

Neural Networks

Dariusz Piatkowski

910 317 430

Abstract - The relation of speaker verification to other pattern recognition problems in speech is discussed, especially the distinction between speaker verification and speaker identification. An experiment is than described which attempts to perform speaker verification with the use of a backpropagation neural network.

1. Introduction

“Pattern-recognition problems are among the most challenging and fascinating areas in speech research. The speech pattern recognition facility of human beings is remarkable. It is easily taken for granted and is only appreciated when one attempts to make machines perform similar tasks” Speech processing inherently involves a large amount of data processing. But as computer hardware has gotten faster and more powerful in the past few years, many of the previously hard problems have become more accessible to research and many of the proposed solutions have also become feasible.

One of such problems is that of speaker-recognition. Although it seems to be on the order of magnitude less difficult than speech recognition, it is nevertheless a difficult and challenging problem. This particular problem can be subdivided into more general and tightly defined sub-problems. These sub-problems are speaker identification and speaker verification. A clear distinction should be made between them.

The more general problem of speaker identification  can be stated as follows: out of a total population of N speakers, find that speaker whose sample reference pattern is most similar to the pattern of an unknown speaker. In this case a sample pattern is compared to a number of reference patterns. Thus there is a finite probability of an incorrect decision for each comparison. The speaker verification problem on the other hand can be stated as follows: a sample speech pattern of an unknown speaker is given, together with a claimed identity. The goal is to determine whether the sample pattern is sufficiently similar to the reference pattern associated with the given speaker. If so the claim is accepted.

In this paper I will discuss the speaker verification aspect of speech pattern recognition. The backpropagation neural network serves as the implementation model for a speaker verification system.

2. Background Information

Much of the theory of speaker verification is common to speaker identification as well. The speaker verification systems are theoretically capable of holding large amounts of speaker data.  This fact has been a driving force for the development of such systems which could be utilized in  applications such as banking and credit authorization, or access to secure information or premises. The two most commonly used techniques for speaker verification deal with the auditory and visual  comparisons of the speech data. The visual comparison of data is usually done through spectrogram analysis or the analysis of other speech features which can be displayed. There remains only one other assumption, and it is that the speakers are cooperative. This means that it is assumed the speaker is not making a conscious effort to alter his speech between the trials.  Due to this fact such a speaker verification system has little effect in forensic applications.

Since the speech data is quite voluminous in nature it is usually pre-processed before actually analyzing it. The pre-processing might involve several techniques. One of the most common ones is the filter bank. It simply passes the speech sample through a “series of contiguous bandpass filters spanning the useful frequency range of speech”. The output of each filter is rectified, smoothed and sampled over a specific time period (usually in the range of ms). This obtains the energy of the signal over that particular passband. Thus filter banks can provide an effective analysis of speech signals. There are a few  areas of interest where the filter bank is not very efficient, such as in the pitch and formant analysis.  In such cases the filter bank processing can be used in combination with other methods such as the above mentioned formant or the LPC  analysis.

Once the speech signal is sufficiently pre-processed we may begin the actual speaker verification task. There are a number of methodologies which can be followed for this process. Some are statistically based, while others are based on the more recently developed models such as hidden Markhov chains, or the connectionist approach. In case of the statistical models, they involve the comparison of specific features of the speech signal. “The most common statistical feature selection technique is that of the F-ratio or analysis of the variance. In this case several statistics are computed over the set of reference utterances provided by each speaker”. Such statistics may include: mean feature vector matrix as well as the covariance matrix of the elements in the feature vector.

In this paper we are concerned with the use of the connectionist model, also known as the neural network model for the task of speaker verification. As such a backpropagation neural network is utilized to perform the final signal analysis. As previously mentioned the speech signal must be first pre-processed in order to minimize the amount of data the neural network must analyze. To perform this pre-processing a speech compression method is used. The method is described by the Global System for Mobile telecommunications (GSM), which was developed by the European Telecommunications Standards Institute in the late 1980s. The compression standard involves the use of a “...Regular Pulse Excited -- Linear Predictive Coder (RPE--LPC) with a Long Term Predictor loop.  Basically, information from previous samples, which does not change very quickly, is used to predict the current sample.  The coefficients of the linear combination of the previous samples, plus an encoded form of the residual, the difference between the predicted and actual sample, represent the signal. Speech is divided into 20 millisecond samples, each of which is encoded as 260 bits, giving a total bit rate of 13 kbps.” Figure 1 illustrates the process a GSM encoder  goes through.

Figure 1

This speech compression is first used to compress a given speech sample by a ratio of nearly 10:1. The resulting data is than further processed (i.e. normalized) before it is finally presented to the backpropagation neural network. A short summary of the neural network architectures follows.

There are many existing Artificial Neural Network (ANN) architectures. Some of them are specialized in their design to solve only the problems dealing with a particular problem domain set. Others on the other hand are more general. One of such ANNs is the backpropagation neural network.  It is a network based on the supervised learning algorithm, where the learning occurs due to error-correction.  In other words, the connection weights between the Processing Elements (PE) are adjusted “in proportion to the difference between the desired and computed values of each PE in the output layer.

where

-  is the error between the calculated and desired value of the output unit ,

-  is the connection strength from  to , and

-  is the learning rate, typically 0<α<<1.”

A typical backpropagation network consists of three layers. There is an input layer, which accepts the input vectors, there is the hidden layer, which basically allows the network to solve non-linear problems, and finally there is the output layer which produces the final network output results.  All the layers are composed of neurodes (the PE) which accept as their input a cumulative total of all the PE outputs from a previous layer, where each single input is being multiplied by a given connection weight positioned at that particular neurode connection before being finally summed up for that given neurode.  The output of a PE is defined by an activation function which may be any of the following:

1. threshold function

2. sigmoid function

3. tanh function

Since we want the activation function to be continuous and differentiable in order to be able to perform gradient descent, we can only use the sigmoid or the tanh function.  They are defined as follows:

1. sigmoid function : f(x)=1/(1+e-x)

2. tanh function : f(x)=tanh(x)

The  backprop network is a feedforward net. It does not have feedback connections, but errors are backpropagated during training. In general a BP network has X input vectors. They represent all the input signals which are to be processed. There also are Y output vectors, and those represent the network outputs. The network may be multilayered and in fact usually utilizes three layers. The first is the input layer, which accepts the above mentioned input vectors. Second is the so-called hidden layer,  which can have any number of neurodes. Finally the third layer is the output layer, which has the Y output vectors. The layers are connected to each other. They are not however fully connected. The connections strictly run only between the outgoing outputs of a lower layer, to the incoming inputs of the higher layer. Thus there are no connections between the first and the third layers. The connections that do exist are between the first and second (hidden) layer, and second and third layer. Other architectures are also possible which implement more network layers. However it has been proven that a three layer network is able to solve a non-linear problem, so the choice between a given network architecture strictly belongs to the designer and is most often derived from previous empirical experience with the given problem. In short, if the experimenter feels that a given problem domain might be better addressed using a more layered network, that is what is built. In most cases though a three layer network is sufficient, and since the full scope of interactions between the various network components is so far not fully understood, a three layer network remains the better choice for all but the most difficult and sophisticated problems. In this case a number of network architectures have been examined.

3. System Design

In this experiment a total of four individual speaker samples were used. Each speaker provided six speech samples. These utterings were based on a previously defined sentences whose objective was to exercise the widest possible human speech spectrum. Therefore the sentences utilized vowels, and consonants in such a way as to produce the most representative speech sample with all the typical characteristics of an everyday pronunciation of the speaker. The following are the sentences used in the experiment:

1. “Even entry-level servers bundle software tools that help you install them the right way, saving you major hassles down the road.”

2. “Scientific reviews from leading industrial and academic labs from around the world.”

3. “Stylishly designed and richly written, a true neo-gothic computer novel.”

4. “You can email the editors of the PC World magazine.”

5. “The difference in compilation speed was like night and day.”

6. “I really enjoyed being a part of this A plus experiment.”

The above sentences were pronounced by each of the speakers and recorded using a speech editing application. Due to the fact that the GSM compression standard was designed for compressing signals of 8kHz  speech frequency rate, and 16-bit per sample sampling rate those were also the recording specifications used for recording the speech samples of the speakers. The actual samples were recorded in a typical household room. This setting was chosen instead of a recording studio room mostly due to the added noise  which would have occurred naturally in any speech sample pronounced in a real world situation.  This way the noise was already factored in once the training samples were presented to the network for learning, avoiding the necessity of artificially introducing such noise. The speech samples were initially recorded in a WAV file format.  Since the application performing the GSM compression  accepted only the AU format files (AU format as used on various Sun machines) the WAV files were converted to an AU file, which was then compressed. The speech frequency as well as the bit/sample rates were preserved in that conversion, thus no signal was lost in the conversion.

Once the file was compressed the result was a GSM compressed sample. Such a sample was composed of a number of 33 byte frames. These bytes represent 76 tightly packed parameters. Each frame, which encodes 160 samples of input, can be subdivided into:

- an initial 8 Log Area Ratios, LARc[0]..LARc[7], which stem from a

short-term analysis of the whole frame (an algorithm similar to

what a regular LPC does),

- followed by four little frames, each for 40 input samples, with

parameters Nc, bc, Mc, and xmaxc, and then 13

values xmc[0]...xmc[12] that make up the encoded residual, that

is, the signal that's left over after being analyzed twice

and having subtracted everything that could be explained using

short-term and long-term linear prediction.

The frames than contain all the required information needed to represent major characteristics of a given speech sample. Next step in the pre-processing is to convert those GSM encoded frames into useable input which can be presented to the neural network for processing.

An input vector to be presented to a neural network must satisfy a few requirements. One of them being its magnitude. Since the network being used is a backpropagation net we define the thresholding function to be a sigmoid function, i.e.

f(x)=11+exp(−x)

This function must have values of x such that the result which will  always

be  in the range of [0,1] is still easily distinguishable (i.e. we want the results

to be as evenly spread in the [0,1] range as possible. To achieve this result we simply normalize the values representing the speech samples. To do this we must find the highest and lowest values present in the given sample, then using these magnitudes we must devise a relation which will map them to our desired range of [0,1].

If we treat a given GSM frame simply as a collection of 33 bytes, we can split the frame into its individual components, the single bytes. Thus in our example we have actual magnitudes which can only be represented by a byte. This means our values present in the frame can only be in the range of [0,255], for a total of 256 values, which is the maximum number of discrete values that can be represented by a byte. Knowing this we now have a range of (0,255) which must be mapped to [0,1]. To perform this mapping we divide a value represented by a single byte in the GSM frame by 255. This normalizes our frame values, which should give us an even distribution of the results in the range of [0,1].

The last step is the preparation of the training data as well as the test data for the neural network. Here we simply repeat the above mentioned process for all the frames in a given speech sample. This produces a file which contains sets of input data and output pattern to associate the data with. The input data consists of 32 values which represent the normalized frame values, the output data depends on the number of speakers the network is being trained to recognize. If there are three speakers we are training on, there are three outputs where each one is a flag for a particular speaker. If we than are preparing training samples for speaker #1, for a network being trained on three speakers, the output pattern will look like the following: 1 0 0 1. The first digit (1) represents the speaker for whom the training data is provided, whereas the last digit, (a 1 in this example) represents the sex of the speaker, which is male in this case. This is performed for all of the samples present for the speakers the network is being trained to recognize.

In the end we have created two files, of which one is the train file and the second is the test file. The train file contains all the training data sets, whereas the test data file contains all the testing data sets. The two files are identical with one exception. The train data file contains the working description of the actual network architecture as well as the values for the various network parameters such as learning rates, number of samples, error tolerance for testing, etc. While training the network the contents of the train file are used as the training data sets. Once the training achieves some particular goal we have defined for it, the test data sets are fed through the network and the overall recall rate is calculated. This is an expression which states the number of correctly identified output vectors out of a total of all the input/output data sets presented to the network for recall. This is expressed as a percentage of correct values out of a total number of test sets present.

As stated above the train file contains a number of parameters. The first line of the train file contains the following in the given order:

#train_sets, #test_sets, #inputs, #outputs, #hidden_layers, #neurons_in_layer1...#neurons_in_last_layer, parameter_flag, input_layer_learning_rate, hidden_layer_learning_rate, sigmoid_function_steepness, momentum, max#of_epochs, min_net_error_for_training, error_tolerance_for_testing.

The parameter_flag simply defines whether the network parameters such as learning rates will be specified in the file or whether they will be supplied by the user during the runtime. This setup allows the user to dynamically create different network architectures for each training session.  Therefore if the user wants to test out a few different architectures with all of them starting at the same point it can be easily accomplished. This is made even easier because the program implementing the backpropagation neural network allows for saving of the original network weights, all the user has to do then is to simply specify an alternative network architecture, start another training session and simply specify the originally saved weights as the new initial weights for this session. A number of different architectures were examined, this was done in order to study the effects of a greater number of hidden layers on the ability of the network to correctly recall speech patterns.

4. Experiment

First the sentences were recorded. The average length of the sentence was as follows:

sentence #1: 6.9 seconds

sentence #2: 4.5 seconds

sentence #3: 4.4 seconds

The sentences were recorded for a total of four speakers, two females and two males. The only editing performed to the original utterances was the deletion of the leading silent portions of the recording (i.e. the recording of the pause before the speaker actually starts talking), and the trailing silent portions of the recording (i.e. the recording of the pause after the speaker actually finishes talking). Additional sentences were also recorded for the purpose of testing the network after a specific learning goal has been achieved. The next step involved the conversion of the WAV file into an AU file. This was accomplished simply by reading the WAV file into a simple sound editing package (GoldWave or CoolEdit) and saving that file using the AU file format. Following this the AU files were than passed through the GSM signal compressor. This application is distributed as a part of the official GSM implementation v10.6 from the Technical University of Berlin. It was compiled under OS/2, using gcc v7.2. The compression process accepts an AU format file and outputs a GSM file which contains the compressed signal in the form of a number of 33 byte long frames. As stated above the GSM file contains the raw compressed speech data which is not suitable as input to the neural network. It must be first normalized.

To normalize that data all the GSM files are passed through another application which reads in a frame at a time, normalizes all the values in the frame and writes them out to an output file. This process not only normalizes the data but also sets up the actual training data sets for the neural network. To perform this, as the user enters the name of the GSM file to be processed a number of other parameters as accepted as well. These parameters are the: identification of the speaker whose speech sample is being processed, the sex of the speaker, the number of speakers whose speech samples will make up the training set, and finally the number of frames which are to be processed. Thus a single GSM frame is converted to 32 inputs which are in magnitude between 0 and 1. The corresponding output vector depends on the identity of the speaker being currently processed, and the sex of the speaker (the sex is represented as a 1 for male speaker and 0 for female speaker). This finally produces a number of train sets which can be presented to the neural network for training. Every speech sample for every speaker produces a separate output file. These files are than combined to form one large file, which contains all the training data sets. The average number of training sets present for each of the sentences were as follows:

sentence #1: 335 sets

sentence #2: 224 sets

sentence #3: 226 sets

The network was trained to recognize as many individual speakers as possible, in this case verification of up to four speakers have been attempted. Three sentences have been chosen as the basis for the training, this being thought sufficient to represent a typical human speech.

A number of architectures were also tested. This was an important consideration since the speaker verification problem is a very difficult problem to solve by its nature. The architectures tested were dictated by a combination of two characteristics: the number of hidden layers, and the total number of neurons in the network. Since we are dealing with 32 inputs, the basic network consisted of only 32 neurons in one hidden layer. The following are the few architectures which were modeled (they only describe the number of neurons in the hidden layers, and the number of hidden layers since we know that we only have 32 inputs, and the size of the output vector varies based on the number of speakers the network is being trained to verify):

#1 - 32 neurons in 1 hidden layer

#2 - 64 neurons in 1 hidden layer

#3 - 96 neurons in 1 hidden layer

#4 - 32 neurons in first hidden layer, 16 neurons in second hidden layer

#5 - 32 neurons in first hidden layer, 32 neurons in second hidden layer

#6 - 64 neurons in first hidden layer, 16 neurons in second hidden layer

#7 - 64 neurons in first hidden layer, 32 neurons in second hidden layer

#8 - 96 neurons in first hidden layer, 8 neurons in second hidden layer

#9 - 96 neurons in first hidden layer, 16 neurons in second hidden layer

#10 - 96 neurons in first hidden layer, 32 neurons in second hidden layer

The training times get progressively longer as the total number of neurons in the network increase. This is not the only reason though, since the network complexity is not only dependent on the overall number of neurons but also the total number of synapses. The number of synapses increases as the network is made up of more hidden layers. On the other hand it can be argued that an increased number of neurons and/or hidden layers should enable the network to have a greater ability to correctly recall a larger set of input/output patterns. As can be seen than this is a situation where a trade off must be made, one between the speed of the learning taking place and the ability of the network to generalize to such a degree as to correctly verify a given speaker to our satisfaction.

A number of training sessions have been devised than to address each one of the above mentioned issues. The network was trained to verify the identity of a single speaker out of a pool of 2, 3, and 4 speakers. The standard training set consisted of training samples generated from 3 sentences pronounced by each speaker. The training sessions were run on a 486DX4/120 AMD CPU computer, under OS/2. The typical training session for up to 2 speakers took in the range of 1000 to 1500 epochs to complete. The training was considered complete when the MSE (mean squared error for the network) was low enough to produce a recall of 80% accuracy with a 10% error tolerance level. The training sessions for 3 and 4 speakers were much longer. They averaged approximately between 4000 to 6000 epochs and above in cases where the training was never completed. The running times for these networks was often in hours and sometimes in days. The number of synapses seemed to have a much greater effect on the length of the training than the number of neurons in the network. The overall results seemed to suggest that the speed of the hardware is still the limiting factor in the training of a backpropagation neural network for the purpose of performing speaker verification. The problem is complicated enough that the use of a large and sophisticated network is warranted, this on the other hand makes necessary the use of fast floating point hardware, large memories and fast secondary storage devices.

5. Experimental Results and Conclusions

When a backpropagation neural network is first initialized the connection weights get assigned random values. At times it can be noticed that when a given problem is very complex in its nature an unlucky assignment of these random values can hinder the learning process of the network. This holds especially true when dealing with networks which are made up of many hidden layers and a large number of neurons. Due to the fact that weight assignment is random and the fact that a solution to such a complex problem is very hard to find, most of these random weight assignments can be considered to fall in the “unlucky” category. In fact generating a set of weights which speeds up the learning of the network is much more unlikely than generating a set of weights that will negatively affect the learning rate of the network at start time.  Since all those initial weights are random, the network goes through a process of “settling the weights”. This phrase describes a situation where the weights may be jumping greatly in value and hardly stabilizing at all. A great indicator of this is the MSE of a network. When the settling takes place the MSE will (sometimes wildly) oscillate between some general range of values.

This oscillation was observed to occur much more frequently in large networks, with large number of neurons. This is to be expected since in a large network we are dealing with many sources of error which is propagated in the early stages throughout the net. What ends up happening is that the initial error adjustments which are backpropagated are very small. Since there are many more synapses (i.e. the connections) which must be affected by these changes the effect is cumulative. In short, a lot more changes, combined with a smaller magnitude of changes leads to a relatively large weight settling period. But once the weights are settled the learning speed appears to match that of other smaller networks. The large networks are not without advantages though. They do seem to have a better ability to generalize, whereas a smaller network will usually overlearn (i.e. start memorizing) the associations between the input/output vectors. In fact a larger network will often produce a comparable recall rate to that of a smaller network when presented with a signal containing more noise, or even signal which was not presented to the network before.

Smaller nets tend to be easier to train. This only seems to hold true for networks which are being trained on a smaller speaker pool, i.e. at most 2 or 3 speakers. Once the total number of speakers increases the single hidden layer networks are usually unable to converge to a solution. No matter how many neurons that single hidden layer might have (this of course is proportionate to the overall sizing of the network).  It was noticed that as the number of neurons was rapidly increased the network was exhibiting the same kind of behavior as a multi hidden layer network. This behavior was not advantageous though, since the single layer network still suffered from typical limitations of a single layer network.  The most obvious behavior which was like that of multilayer networks was the initial oscillation of the MSE. The oscillation usually tended to finish off earlier than a multilayer network would have, with a resulting overall speedup of convergence. This behavior was only obvious when training on a pool of up to 2 speakers. If the network had a bigger pool than that, the training slowed significantly, and in fact the oscillations would continue for an even much greater length of time as compared to a relatively same sized multilayer network.

The actual experimental results are presented below. They show the success rate in verifying a speaker sample, where speakers 1 & 3 are male and speaker 2 is female:

1) 2 hidden layer network with 64 neurons in first layer and 16 neurons in the second layer:

SPEAKER	SAMPLE #1	SAMPLE#2 	SAMPLE#3

1		40.2		42.3		40.1

2		42.5		40.6		41

3		69.7		68		70.5

This was a typical performance of a multilayer network. The samples presented to the network for testing were not seen before, i.e. they were not a part of the training data set.

2)  single hidden layer network with 32 neurons in the hidden layer:

SPEAKER	SAMPLE #1	SAMPLE #2	SAMPLE #3

1		30		35.2		33.8

2		31		32.3		31.4

3		45.2		51.6		56.2

This was a typical performance of a single hidden layer network. The samples presented to the network for testing were not seen before, i.e. they were not a part of the training data set. The performance was similar when a smaller pool of speakers was used to train the network. The only noticeable difference was the lower training time.

Overall than it was obvious that a multilayer network was a much better suited architecture for this kind of a problem. The ideal network would have a large number of neurons, sufficient in fact to be able to separate all the distinguishing characteristics of different speakers. This is a difficult task since human speech often has many similarities when comparing speech samples of different speakers. This can be easily seen when looking at a spectrogram of speech of two different speakers. There are many similarities there, yet we are able to distinguish the speech based on the few characteristics which are peculiar to the speech of the individual speaker. The correct verification results as can be seen are rather low, which would suggest that the complexity of the problem is even greater than suspected. It should be noted that the results for the third speaker were consistently higher. This would tend to suggest perhaps a coding mistake, or a design mistake with regards to the implementation of the backpropagation neural network. Such a possibility was thoroughly examined, but no mistakes were found in the process. The algorithm in fact can be seen working perfectly with many other problems, which use more than 3 or 4 outputs.

6. Summary

It can be seen that the speaker verification problem is very sophisticated and difficult to solve. In this experiment an attempt has been made at using a backpropagation neural network to perform the task of speaker verification. A number of architectures have been examined, many multiple hidden layer, as well as single hidden layer networks have been examined. The results are somewhat disappointing, with a typical verification rate of less than 50%. The better than average performance of the third speaker tends to suggest an underlying trend in the data which might be facilitating the favorable training for that speaker. It must be noted that since the data which the network was trained on was not raw sound data there were many inconsistencies introduced while at the pre-processing stage. For one the GSM speech compression standard is a lossy compression implementation.

Overall several conclusions can be drawn from the experiment. First the speaker verification problem is very difficult as stated above. The amount of similarities between the speech samples of different speakers make it much more difficult to clearly distinguish the data since we can no longer attempt this simply by looking for major differences in the patterns. They must be analyzed at a much deeper level. Such an analysis can be performed by a neural network. Such a network though, due to the nature of the problem must be large. The most suitable architecture seems to be that of multi hidden layer network, with possibly more than 2 or even 3 hidden layers. The training times will be very long, although better pre-processing of the raw speech data might allow for a faster convergence of the network. In short the problem can be solved, but it will require great amount of computational power in order to make it feasible in real time.

Automatic Speech & Speaker Recognition, IEEE Press, “Automatic Speaker Verification: A Review”, Rosenberg A., p.336

Ibid. p.337

Ibid. p.337

Ibid p.337

Ibid p.337

Ibid p.338

Ibid p.338

Ibid p.338

Overview of the Global System for Mobile Telecommunications,  John Scourias, University of Waterloo

Ibid.

Ibid.

“Neural Networks”, unpublished graduate course notes - personal communication,

Dr. Deborah Stacey

C++ Neural Networks and Fuzzy Logic, Rao B., Rao V., MIS Press, New York, 1993, p. 103

GSM Standard, Electronic communications